SYNAPSE
Mission Edge Cases & Risk Assessment

1. Boot & Environment

Things to consider:

- how the Pi boots at -45/-50 C

- kernel modules loading

Overview:

1. ESP32 (OBC) fires first (under 50 milliseconds). It activates the hardware/software watchdogs, initializes sensor loops, and handles telemetry. If the rasp pi dies, the ESP keeps downlinking telemetry to the ground station.

2. Rasp Pi takes around 20-30s too boot. It has to initialize the filesystem and hardware buses of the Linux OS

By default, Linux has no idea what Akida is so we inject the driver akida-pcie.ko into the running Linux kernel, creating a hardware interface file in the system /dev/akida0. The hardware communicates with the SDK through reading and writing of bytes.

Risks

1. Temperature Bug - Fixed

In Synapse\subsystems\obc\esp32-obc\main\tasks\mode_manager_task.c:

bool is_critical = (battery < 3.2f ¦¦ temp.temperature_external_c > 70.0f ¦¦ temp.temperature_external_c < -10.0f); if (is_critical) return MODE_SAFE;

In the stratosphere, external air temperature drops to -40 to -60 C. During ascent, the ESP32 will think the spacecraft is dying and permanently trigger Safe Mode, disabling photo_active = false and aborting camera/inference ops.

Recommendation: The threshold should be modified for internal temp instead.

Source: documentation

"Safe Mode - Any temperature sensor > 50◦C, or Vbat < threshold for 10s"

2. Missing Processing Command

In Synapse\subsystems\obc\rpi\comm\testint_comm.py and Synapse\subsystems\obc\esp32-obc\main\tasks\mode_manager_task.c:

Right now neither scripts know about the run_entire_data_processing_mode.py. When balloon is in active descent mode_manager_task.c should launch a CMD|START_PROCESSING over UART to launch the 4-step AI inference pipeline. testint_comm.py receives the command in rasp pi and launches the python script.

Also OBC side is missing the radio downlink pipeline to receive and broadcast the classification results.

Source: documentation

"Data Processing Mode- Free fall detection (9.8 m/s2) + GPS + Barometer."

3. Thermal Risk  - Ignored

Unless there is a hardware relay that is missing from the software, the Raspberry Pi is hardwired to main power and remains powered on the entire flight, only the software remaining off/idle.

If the raspberry pi is kept powered off between 50m and 25km, the components in the payload might cold-soak. When the ESP32 switches power on to the Pi at 25km:

- MicroSD flash memory controllers might refuse to initialize or write below -20 C

- The onboard oscillator crystal can drift, corrupting PCIe and UART bus clocks

Recommendation: A cold-box freezer boot test (powering pi on after it has sat in a freezer for 1 hour) can be performed.

If only the flight software is kept off/idle Linux will consume around 3 Watts which will act as a steady electrical heater, preventing the bay, batteries, and SD card from reaching -40 C. In this case the battery capacity needs to be checked to cover the energy used (3W at 5V is 600mA, over 2.5 hrs of flight that is around 1500 mAH) plus the Akida inference burst.

2. In-Flight Image Aquisition

Things to consider:

- auto exposure at high altitude

- motion blur when swinging

- check SD storage

Overview:

Hardware: Raspberry Pi Camera Module v2 (Sony IMX219 sensor, 3280 × 2464 resolution)

1. Runs auto_select_shutter() at startup to pick the sharpest shutter speed and analog gain.

2. Enters a while True continuous capture loop, saving a photo every 2 seconds to ~/Desktop/foto_mode/poze/.

3. Appends photo metadata, camera settings, and GPS coordinates to log_pozitii.csv.

Risks

1. Auto Exposure + more testing here -> FACUT

In Synapse\subsystems\payload\src\Code on Rasp Pi\foto_mode\script.py:

auto_select_shutter() is ran once at boot, then never again because of the AeEnable = false. At ground the algorithm might pick a slow shutter and boost the gain, but at high altitudes solar irradiance is 15% stronger and cloud decks have an higher albedo.

Recommendation: Either re-run exposure calibration at altitude, or enable Auto Exposure

2. Motion Blur - Done

If the balloon will swing and oscillate and shutter speed is slower than 1/500s, angular rotation might blur roads, rivers, and vegetation across multiple pixels

Recommendation: Hard ceiling on exposure time: shutter must be <= 1/1000s to freeze motion blur

Other things to consider: adding an exit condition in script.py alongside CTRL+C, locking white balance to daylight ("AwbMode": "daylight") to stop different tints from payload oscillation, adding a try/except to picam2.start() to prevent crashes in case of a voltage drop or cable noise.

3. Candidate Selection & Processing

Things to consider:

- popping early

- unhandled corrupted image crash

Overview:

1. Candidate Selection (choosing_foto.py):

Filters log_pozitii.csv for photos taken above 25 km (MIN_ALT_M = 25000.0).

Picks up to 20 candidate photos that maximize geographical spread (using Convex Hull and Haversine distance).

Evaluates each candidate through a Clarity Filter (checks extreme exposure, cloud deck, cloud quadrants, blur, and atmospheric haze).

Selects the top 5 clearest photos and copies them into ~/Desktop/data_processing_mode/photos_for_akida/.

2. Preprocessing (pre-processing.py):

Pads each image with black pixels to reach exact multiples of 64 (3280 × 2464 ➔ 3328 × 2496).

Slices each image into a grid of 52 × 39 = 2,028 patches of 64 × 64 pixels.

Stacks all 2,028 patches in RAM into a single 4D NumPy array (2028, 64, 64, 3) and saves it as batch.npy.

Risks

1. Hard Altitude Ceiling Risk - Done

We need to consider what if the balloon pops earlier than expected (like 24800 meters), or the GPS altitude calibration has a small offset. In that case, choosing_foto.py will find 0 photos, log a message and exit without selecting any photos - mission loss of AI results.

if not high_alt: print("Nicio poza nu indeplineste pragul de altitudine.") return

Recommendation: A fallback is needed, something like: if no photos are above 25k km, select top 20 photos with highest recorded altitude

2. Unhandled Corrupted Image Crash - Done

In pre-processing.py:

img = Image.open(image_path).convert("RGB")

If a photo write was cut short by a power fluctuation or SD write stall, PIL throws an unhandled PIL.UnidentifiedImageError or OSError: image file is truncated. There is zero try/except inside process_image(). If one single photo out of the 5 is corrupt, pre-processing.py crashes on that photo and aborts the entire preprocessing run, generating zero segments for the remaining 4 perfectly good photos.

Recommendation: again we should wrap image opening in try/except (OSError, Image.DecompressionBombError): log the corrupted photo and continue slicing the remaining ones.

4. Edge AI Inference

Things to consider:

- pcie stability

- fragile $home paths

- unhandled exception in np.load(batch.npy)

- obsolete else

Overview:

1. Host Driver & Container Launch (start_akida.sh):

Verifies the kernel module is loaded (/dev/akida0); injects akida-pcie.ko if missing.

Spins up the isolated Docker container (akida-image) with direct PCIe hardware access (--device=/dev/akida0).

Executes inference.py inside the container.

2. Hardware Inference Pipeline (inference.py):

Loads the trained 8-class SNN model: eurosat_8classes_v1.fbz.

Calls map_to_hardware() to explicitly bind the model to /dev/akida0 .

Loads batch.npy (all 2,028 patches per photo) and runs bulk batch inference via model.predict_classes(flight_batch).

Writes dual outputs:

.csv for complete on-board SD card archiving.

.bin (2,028 uint8 bytes) for LoRa radio downlink.

Logs overall performance (FPS, latency, frame count) to log_performanta.csv.

Risks:

1. PCIe Link Stability

During balloon burst and high-speed descent, the payload experiences tumbling and vibration. Because of this, thermal contraction on PCIe Gen 2/3 lanes can cause PCIe link resets or DMA timeouts in the akida_pcie driver.

Recommendation: The ribbon cable should be mechanically secured with tape/foam padding so physical shock during balloon burst doesn't disrupt the bus pins.

2. Fragile $home paths

In start_akida.sh, if this script is launched automatically at boot via systemd or /etc/rc.local, $HOME resolves to /root, not /home/synapse2025. The script immediately crashes with "Nu gasesc folderul /root/akida-setup/akida_dw_edma".

Recommendation: Hardcode absolute paths (e.g. /home/synapse2025/Desktop/...) instead of relying on $HOME, and configure the user in /etc/sudoers with NOPASSWD: ALL so docker and insmod never prompt for a password.

3. Unhandled Exception on np.load(batch.npy) - Done

In inference.py:

There is zero try/except block around np.load(batch_path) or model.predict_classes(). If the balloon bursts or power fluctuates while pre-processing.py is writing batch.npy to the SD card, the file will be partially written or corrupted. When inference.py hits that corrupted file, np.load throws an uncaught ValueError / EOFError, crashing the entire Python script immediately.

Recommendation: We should wrap the batch inference loop in a try/except Exception as e: so a single bad file is logged and skipped, allowing the remaining 4 images to be classified.

4. The Obsolete Else - Done

In inference.py:

In inference.py, if batch.npy is missing, the code falls back to an else branch that looks for individual Frame_N.png files. However, in pre-processing.py, the code that saves .png files was commented out to save CPU cycles.

We should just remove this else branch.

5. Downlink Telemetry

Things to consider:

- lora packet size

- missing packet shift

Overview:

1. Onboard Binary Serialization (inference.py):

Directly converts Akida prediction indices into an 8-bit unsigned integer array:

predictions.astype(np.uint8).tofile(bin_path)

Exactly 2,028 bytes per image (52 × 39 patches), replacing the 46.6 KB plain text CSV.

2. Ground Station Telemetry Decoding (decode_downlink.py):

Reads the downlinked .bin file, matches each byte index to its corresponding class name, computes class percentage distributions, and outputs a ground-verified CSV table.

Risks:

1. Lora Packets

A standard LoRa transceiver (Semtech SX1276 / SX1262) cannot transmit 2,028 bytes in one single message.

The maximum physical hardware packet limit (MTU) for LoRa is 255 bytes.

In practice, payload size per packet is typically kept to ~128–200 bytes for radio stability. If the OBC tries to send foto_1.bin (2,028 bytes) as one continuous chunk, the LoRa radio driver will reject it or truncate it to 255 bytes, losing ~88% of the data.

Recommendation: The binary file must be packetized into smaller radio packets (2028 bytes / 200 bytes/packet = 11 packets per photo)

2. Missing Packet Shift

In decode_downlink.py:

raw_bytes = list(bin_path.read_bytes()) decoded_labels = [CLASSES[b] ... for b in raw_bytes]

The decoder assumes byte index i strictly corresponds to patch index i on the 52 × 39 spatial grid but if packet #3 (patches 400 to 600) is dropped in the radio link due to interference, and the ground station simply concatenates the surviving packets: byte 601 now sits at index 401

Every subsequent patch shifts backwards. The entire 2D reconstructed map of Earth gets corrupted: forests end up in rivers, and highways end up in crops.

Recommendation: Each LoRa packet must include an offset header [PHOTO_ID, PACKET_NUM, OFFSET]. The ground receiver must initialize a fixed 2,028-byte array (pre-filled with 0xFF / Unknown) and place incoming bytes into their exact grid slots.

DE FACUT PIPELINE DE TESTARE ca lumea:

- Testare captura de imagine ca lumea: ceata, alte conditii

- Pipeline end to end de testare -> de terinat, full, nu pe bucati;

- De sters chestiile din sistemul de operare ->

- Mod manual: cum putem face -> sa fie mereu receptor?