### Architectural Flow:

#### 1. Initialization

The model initialization happens inside a Docker container, which is granted hardware access via the --device=/dev/akida0 flag. The Akida kernel module itself is handled by host system commands (sudo insmod ./akida-pcie.ko) before Docker starts.
 The camera code is decoupled from the inference script. The camera runs on the host Raspberry Pi OS, snapping photos and automatically dropping them into /home/synapse2025/Desktop/akida_sample folder.

#### 2. Preprocess

Pass the photo to the prepare_flight_batch() function. The script will use Pillow to load the image file from /data/, convert it to a NumPy array, and pass it directly into prepare_flight_batch(image) to yield the 4D tensor stack.

#### 3. Hardware InferenceFlight

Pass the 4D batch resulted from preprocessing into model_akida.predict_classes(). The inference must be wrapped in high-precision time deltas.

#### 4. Output

Here the scripts infer.py and infer_all.py are execution runs that calculate and print out final performance reports directly to the terminal console. The script needs to calculate and format these exact metrics:

- Total images processed

- Correctly classified patches vs. Incorrectly classified patches

- Global accuracy percentage

- Total processing time (seconds)

- Average time per image (milliseconds)

- Total frames per second

### Testing

#### Results on infer.py for baseline_model.fbz

Model Summary

Input shape: [64, 64, 3]
 Output shape: [1, 1, 10]
 Sequences: 1
 Layers: 14

Layer (type) Output shape Kernel shape

========== SW/stem_conv-dequantizer (Software) =========

stem_conv (InputConv2D) [32, 32, 32] (3, 3, 3, 32)

b1_dw (DepthwiseConv2D) [32, 32, 32] (3, 3, 32, 1)

b1_pw (Conv2D) [32, 32, 64] (1, 1, 32, 64)

b2_dw (DepthwiseConv2D) [16, 16, 64] (3, 3, 64, 1)

b2_pw (Conv2D) [16, 16, 64] (1, 1, 64, 64)

b3_dw (DepthwiseConv2D) [16, 16, 64] (3, 3, 64, 1)

b3_pw (Conv2D) [16, 16, 128] (1, 1, 64, 128)

b4_dw (DepthwiseConv2D) [8, 8, 128] (3, 3, 128, 1)

b4_pw (Conv2D) [8, 8, 128] (1, 1, 128, 128)

b5_dw (DepthwiseConv2D) [8, 8, 128] (3, 3, 128, 1)

b5_pw (Conv2D) [1, 1, 256] (1, 1, 128, 256)

fc1 (Dense1D) [1, 1, 128] (256, 128)

logits (Dense1D) [1, 1, 10] (128, 10)

dequantizer (Dequantizer) [1, 1, 10] N/A

## Total Patches: 510 | FPS: 174.55 | Avg Latency: 5.73ms

[AnnualCrop] -> Count: 4 (0.8%)
 [HerbaceousVegetation] -> Count: 78 (15.3%)
 [Highway] -> Count: 35 (6.9%)
 [Industrial] -> Count: 154 (30.2%)
 [PermanentCrop] -> Count: 4 (0.8%)
 [Residential] -> Count: 229 (44.9%)
 [River] -> Count: 5 (1.0%)
 [SeaLake] -> Count: 1 (0.2%)

#### Results on infer.py for attuned_model.py:

Model Summary

Input shape: [64, 64, 3]
 Output shape: [1, 1, 10]
 Sequences: 1
 Layers: 8

Layer (type) Output shape Kernel shape

=========== SW/stem_conv-logits (Software) ==========

stem_conv (InputConv.) [32, 32, 32] (3, 3, 3, 32)

b1_sep (Sep.Conv.) [32, 32, 64] (3, 3, 32, 1)

(1, 1, 32, 64)

b2_sep (Sep.Conv.) [16, 16, 64] (3, 3, 64, 1)

(1, 1, 64, 64)

b3_sep (Sep.Conv.) [16, 16, 128] (3, 3, 64, 1)

(1, 1, 64, 128)

b4_sep (Sep.Conv.) [8, 8, 128] (3, 3, 128, 1)

(1, 1, 128, 128)

b5_sep (Sep.Conv.) [1, 1, 256] (3, 3, 128, 1)

(1, 1, 128, 256)

fc1 (Fully.) [1, 1, 128] (1, 1, 256, 128)

logits (Fully.) [1, 1, 10] (1, 1, 128, 10)

## Total Patches: 510 | FPS: 286.47 | Avg Latency: 3.49ms

[AnnualCrop] -> Count: 8 (1.6%)
 [HerbaceousVegetation] -> Count: 19 (3.7%)
 [Highway] -> Count: 44 (8.6%)
 [Industrial] -> Count: 104 (20.4%)
 [PermanentCrop] -> Count: 33 (6.5%)
 [Residential] -> Count: 289 (56.7%)
 [River] -> Count: 12 (2.4%)
 [SeaLake] -> Count: 1 (0.2%)

#### Results on infer_all.py for baseline_model.fbz:

Se ruleaza inferenta...

[AnnualCrop] 2901/3000 = 96.70% | timp: 132.06s
 [Forest] 2973/3000 = 99.10% | timp: 125.68s
 [HerbaceousVegetation] 2886/3000 = 96.20% | timp: 122.45s
 [Highway] 2438/2500 = 97.52% | timp: 100.73s
 [Industrial] 2464/2500 = 98.56% | timp: 102.64s
 [Pasture] 1881/2000 = 94.05% | timp: 80.25s
 [PermanentCrop] 2264/2500 = 90.56% | timp: 99.83s
 [Residential] 2985/3000 = 99.50% | timp: 121.54s
 [River] 2372/2500 = 94.88% | timp: 99.73s
 [SeaLake] 2985/3000 = 99.50% | timp: 137.93s

REZULTATE FINALE
 Total imagini procesate : 27000
 Clasificate corect: 26149
 Clasificate gresit: 851
 Acuratete globala: 96.85%
 Timp total: 1123.37s
 Timp mediu per imagine: 41.61ms
 Imagini per secunda: 24.0 FPS

#### Results on infer_all.py for attuned_model.fbz:

Se ruleaza inferenta...

[AnnualCrop] 2822/3000 = 94.07% | timp: 60.40s
 [Forest] 2606/3000 = 86.87% | timp: 62.67s
 [HerbaceousVegetation] 2759/3000 = 91.97% | timp: 59.20s
 [Highway] 2452/2500 = 98.08% | timp: 50.80s
 [Industrial] 2424/2500 = 96.96% | timp: 58.09s
 [Pasture] 1884/2000 = 94.20% | timp: 43.08s
 [PermanentCrop] 2381/2500 = 95.24% | timp: 51.13s
 [Residential] 2989/3000 = 99.63% | timp: 64.97s
 [River] 2410/2500 = 96.40% | timp: 60.90s
 [SeaLake] 2968/3000 = 98.93% | timp: 62.59s

REZULTATE FINALE
 Total imagini procesate : 27000
 Clasificate corect: 25695
 Clasificate gresit: 1305
 Acuratete globala: 95.17%
 Timp total: 574.10s
 Timp mediu per imagine: 21.26ms
 Imagini per secunda: 47.0 FPS

#### Results on infer_all.py for attuned_model.fbz after data augmentation:

Se ruleaza inferenta...

[AnnualCrop] 66/100 = 66.00% | timp: 2.50s
 [Forest] 4/100 = 4.00% | timp: 2.16s
 [HerbaceousVegetation] 50/100 = 50.00% | timp: 2.00s
 [Highway] 47/100 = 47.00% | timp: 1.99s
 [Industrial] 70/100 = 70.00% | timp: 2.00s
 [Pasture] 21/100 = 21.00% | timp: 1.98s
 [PermanentCrop] 25/100 = 25.00% | timp: 2.07s
 [Residential] 41/100 = 41.00% | timp: 1.99s
 [River] 48/100 = 48.00% | timp: 1.97s
 [SeaLake] 89/100 = 89.00% | timp: 2.02s

REZULTATE FINALE
 Total imagini procesate : 1000
 Clasificate corect : 461
 Clasificate gresit : 539
 Acuratete globala : 46.10%
 Timp total : 20.77s
 Timp mediu per imagine : 20.77ms
 Imagini per secunda : 48.2 FPS

Conclusion:
 After noise injection the model is practically blind to forests now. In the clean dataset, forests are easily indentified by deep, dark green colors and textures (leaves). Here, Rayleight Scaterring washed away the deep green, replacing it with blue/gray, the blur decreased texture quality and once these are gone, the model has no geometric shapes to latch onto.
 It seems like Pasture and PermanentCrop both suffered like forest. These classes rely heavily on distinct color shades and ground patterns. When introduced to sun glares and clouds, pastures look exactly like HerbaceousVegetation or AnnualCrop.
 This proves that the original model didn't actually learn what a forest is. It learned what a perfect low-altitude photo of a forest looks like.