SYNAPSE

Stratospheric Balloon Mission

Launch: Mid-September

TASK DOCUMENT

Payload SW Integration

Task Deadline: 5 July

Responsible: Mirel, Violeta

Mission: SYNAPSE

Document Version: 1.0

Status: DRAFT

CLASSIFICATION: INTERNAL

# Table of Contents

# 1. Scope

This document covers the software integration of the Raspberry Pi 5 and the Akida BrainChip neuromorphic chip for the SYNAPSE payload. It includes the physical connection setup, environment configuration, inference pipeline development, camera integration, and performance benchmarking.

## 1.1 Objectives

- Set up the Raspberry Pi — Akida hardware and software stack.

- Run inference on the Akida chip using EuroSAT satellite images.

- Develop and validate a full pipeline: image capture → segmentation → inference.

- Benchmark performance under realistic conditions.

- Iterate the model based on real conditions.

## 1.2 Out of Scope

- Payload hardware assembly (covered by hardware task).

# 2. Requirements

## 2.1 Functional Requirements

- The system shall run inference on the Akida chip.

- The inference pipeline shall classify 64x64 image patches from segmented satellite images.

- The system shall segment large images into 64x64 patches before inference.

- The full pipeline (capture → segment → infer) shall be implemented.

## 2.2 Non-Functional Requirements

- Inference time per image and per full batch shall be measured and documented.

- The pipeline shall be tested with real satellite images photographed from a screen (realistic conditions) and various satellite images.

# 3. Implementation

## 3.1 Implementation Steps

- Test inference on Akida: classify 100 EuroSAT images saved locally on the Raspberry Pi. Measure classification accuracy and time. Then test on all images and record total inference time.

- Download a large satellite image from the internet. Run the segmentation script on it. Classify the resulting patches. Measure time and performance.

- Integrate the camera: configure it to capture images and save them to the Raspberry Pi automatically.

- Develop the segmentation script: split large images into 64x64 patches.

- Photograph satellite images from a screen and run the full pipeline (segmentation → inference). Record performance parameters.

- Iterate the model taking into account the most realistic operating conditions.

## 3.2 Physical Setup — Raspberry Pi & Akida Connection

Connect the Raspberry Pi to a laptop via UART using the red USB communication module. Wire connections as shown in the images below.

Physical wiring — Raspberry Pi to UART module (1/3)

Physical wiring — Raspberry Pi to UART module (2/3)

Physical wiring — Raspberry Pi to UART module (3/3)

Raspberry Pi and Akida board assembled

## 3.3 Driver Installation & Serial Connection (UART)

### Step 1 — Install UART driver

Download and install the VCP driver for the red USB communication module:

FTDI VCP Drivers: https://ftdichip.com/drivers/vcp-drivers/

Run the setup executable, install, then restart the laptop.

Driver installation — setup executable

### Step 2 — Identify COM port

Connect the red module via USB. Open Device Manager and note the COM port number under Ports:

Device Manager — COM port identification

### Step 3 — Connect via PuTTY

Download PuTTY: https://putty.org/index.html

Open PuTTY and configure as follows: Connection type: Serial | Speed (Baud rate): 115200 | Serial line: COM5 (or your identified port).

PuTTY configuration

Connect the wires from the Raspberry Pi to the red module while the Pi is powered on. You should now have a serial terminal.

Login credentials:

Hostname: synapse2025

Password: synapse2025

### Step 4 — WiFi connection

Connect to the WiFi hotspot using the following command:

sudo nmcli dev wifi connect "My" password "violetamihai"

WiFi hotspot credentials: SSID: My | Password: violetamihai (rename phone hotspot accordingly if needed).

## 3.4 Akida Driver & Docker Environment

### Step 1 — Load Akida kernel module

Run the following each time the Raspberry Pi boots:

cd ~/akida-setup/akida_dw_edma

sudo insmod ./akida-pcie.ko

### Step 2 — Verify Akida device

ls -l /dev/akida*

Expected output:

crw------- 1 root root 10, 260 Jun  9 00:57 /dev/akida0

### Step 3 — Dockerfile

Create the Dockerfile on the Raspberry Pi:

nano ~/akida-setup/test-simple-program/Dockerfile

Content:

FROM ubuntu:22.04

RUN apt update && apt install -y python3 python3-pip

RUN pip3 install akida==2.16.1 numpy==1.26.4 Pillow

WORKDIR /data

Save with Ctrl+X → Y → Enter, then build the image:

sudo docker build -t akida-image ~/akida-setup/test-simple-program/

### Step 4 — Start the container

sudo docker run -it --device=/dev/akida0 -v /home/synapse2025/Desktop/akida_sample:/data akida-image

### Step 5 — Transfer images to Pi

From Windows laptop, copy images to the Raspberry Pi:

scp -r "C:\Users\Violeta\Desktop\Synapse\subsystems\payload\images" synapse2025@192.168.137.189:/home/synapse2025/Desktop/akida_sample/

## 3.5 Startup Checklist (Every Session)

Follow these steps each time the Raspberry Pi is powered on:

- Load the Akida kernel module:

cd ~/akida-setup/akida_dw_edma

sudo insmod ./akida-pcie.ko

- Verify the device exists:

ls /dev/akida*

- Start the Docker container:

sudo docker run -it --device=/dev/akida0 -v /home/synapse2025/Desktop/akida_sample:/data akida-image

### Inside the container

Run inference on a single image:

python3 /data/infer.py

Run inference on all images:

python3 /data/infer_all.py

Exit the container:

exit

### Key files

- /home/synapse2025/Desktop/akida_sample/infer.py — inference on a single image

- /home/synapse2025/Desktop/akida_sample/infer_all.py — inference on all images across 10 classes

- ~/akida-setup/test-simple-program/Dockerfile — Docker container recipe

Note: files in /data/ inside the container map directly to /home/synapse2025/Desktop/akida_sample/ on the Pi. Scripts edited on the Pi are immediately visible in the container. Docker only needs to be rebuilt if the Dockerfile changes.

Performances

## 3.6 Performance Results

[Fill in inference results as tests are completed.]

| Test | Images | Time (total) | Notes |
| --- | --- | --- | --- |
| EuroSAT — 100 images | 100 | [TBD] |  |
| EuroSAT — full dataset | [TBD] | [TBD] |  |
| Large image — segmented | [TBD] | [TBD] |  |
| Full pipeline (screen photo) | [TBD] | [TBD] | Real conditions |

# 4. Risks and Mitigation

| ID | Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| RSK-01 | Akida kernel module fails to load after reboot | Medium | High | Load kernel module manually each session (sudo insmod ./akida-pcie.ko). Document startup checklist clearly. |
| RSK-02 | Docker container loses access to /dev/akida0 | Medium | High | Always pass --device=/dev/akida0 flag when starting container. Verify device exists before running container. |
| RSK-03 | Inference performance insufficient for real-time pipeline | Medium | Critical | Benchmark early (Section 3.1). Iterate model and segmentation parameters if needed (Section 3.5). |
| RSK-04 | Camera integration fails or produces incompatible image format | Low | Medium | Test camera save pipeline independently before integrating with segmentation + inference scripts. |
| RSK-05 | WiFi credentials change or hotspot unavailable during setup | Low | Medium | Document current credentials. Fallback: use UART/USB connection as alternative access method. |

## 4.1 Additional Notes

[Any additional risk observations or mitigation actions not captured in the table above.]